# Design of Sterically Hindered Monomer-Incorporated Biodegradable Polyesters for Enhanced Thermal Resistance without Compromised Degradation Kinetics

## Abstract

This study proposes a biodegradable polyester design incorporating sterically hindered monomers to elevate glass transition temperature (Tg) while preserving degradation kinetics. Mechanistic evidence indicates that rigid bicyclic structures (e.g., isosorbide) restrict chain mobility, increasing Tg by 40-100°C in aliphatic polyesters [1, 2, 3]. However, literature reveals an inherent trade-off: Tg elevation suppresses enzymatic hydrolysis rates due to reduced chain mobility in amorphous domains [4, 5]. The experimental route involves synthesizing poly(butylene succinate-co-isosorbide succinate) copolymers with 0-40 mol% isosorbide, characterizing thermal/structural properties, and evaluating degradation under enzymatic/hydrolytic conditions. Key risks include suppressed degradation at high isosorbide loadings and molecular weight limitations [3]. Preliminary evidence suggests optimal performance may occur at intermediate compositions (10-20 mol% isosorbide), balancing thermal and degradation requirements.

## 1. Introduction

### 1.1 Scientific Background

Biodegradable polyesters like poly(butylene succinate) (PBS) face limitations in high-temperature applications due to low Tg (-30°C to 0°C) [1, 6]. Enhancing heat resistance typically suppresses degradation kinetics through increased crystallinity or reduced chain mobility [4, 5], creating a fundamental design challenge.

### 1.2 Design Hypothesis

Incorporating sterically hindered monomers (e.g., isosorbide) will increase Tg via restricted chain mobility while maintaining biodegradability through hydrolytically labile ester bonds. The rigidity-degradation trade-off can be mitigated by optimizing monomer stoichiometry to avoid excessive crystallinity.

### 1.3 Scope of This Study

Direct evidence supports Tg elevation using isosorbide [1, 2, 3] and degradation suppression by high Tg/T_m [4, 5]. Experimental validation is required for: 1) Copolymer degradation kinetics, 2) Optimal isosorbide content balancing Tg and degradation, and 3) Melt-processability of high-Tg compositions.

## 2. Mechanistic Rationale and Design Hypothesis

### 2.1 Mechanistic Basis

Sterically hindered monomers like isosorbide introduce conformational rigidity, restricting segmental motion and elevating Tg by 40-100°C in aliphatic polyesters [1, 2, 3]. Isosorbide-based polyesters achieve Tg up to 146°C while remaining biodegradable [3]. Degradation occurs preferentially in amorphous regions via enzyme-mediated hydrolysis, with kinetics inversely correlated to Tg due to reduced chain mobility limiting enzyme-substrate complex formation [4, 5].

### 2.2 Evidence-Supported Design Rules

- **Thermal Properties**: Tg increases linearly with isosorbide content [1, 2, 3]

- **Degradation**: Hydrolysis rate decreases exponentially with Tg elevation [4, 5]

- **Biocompatibility**: No direct evidence; requires experimental validation

- **Structural Control**: Crystallinity modulation via copolymerization critical for degradation kinetics [5, 6]

### 2.3 Trade-Offs and Constraints

High isosorbide content (>30 mol%) elevates Tg but may: 1) Reduce molecular weight during synthesis [3], 2) Increase crystallinity, slowing degradation [4], and 3) Narrow melt-processing windows. Optimal compositions likely balance rigidity with sufficient chain mobility for enzymatic access.

## 3. Materials and Methods

### 3.1 Material System and Variable Definition

- **Base System**: Poly(butylene succinate-co-isosorbide succinate)

- **Primary Variable**: Isosorbide molar fraction in diol component (0, 10, 20, 30, 40%)

- **Fixed Components**: Succinic acid (100 mol%), catalyst (titanium butoxide)

- **Unknowns**: Optimal copolymer sequence distribution, crystallization kinetics

### 3.2 Formulation / Sample Matrix Design

| Sample | Isosorbide (mol%) | Butanediol (mol%) | Purpose |

|--------|-------------------|-------------------|---------|

| PBS   | 0 | 100 | Baseline control |

| PIS10 | 10 | 90 | Low-hindrance test |

| PIS20 | 20 | 80 | Target balance zone |

| PIS30 | 30 | 70 | High-hindrance test |

| PIS40 | 40 | 60 | Degradation risk assessment |

### 3.3 Sample Preparation / Fabrication Procedure

1. **Monomer Mixing**: Charge succinic acid, butanediol/isosorbide mixture (1.05:1 diol:diacid ratio), and 200 ppm Ti(OBu)_4

2. **Esterification**: 180°C under N_2 for 2 hr (water removal monitored)

3. **Polycondensation**: 220°C at <1 mbar for 3 hr with mechanical stirring

4. **Quenching**: Rapid cooling in liquid N_2 to minimize crystallization

5. **Annealing**: Subset samples at T_m-20°C for crystallinity control

### 3.4 Structural and Physicochemical Characterization

- **Molecular Weight**: GPC (DMF, 40°C) for M_n/M_w

- **Thermal Analysis**: DSC (10°C/min, N_2) for Tg, T_m, ΔH_f

- **Crystallinity**: XRD and DSC crystallinity calculations

- **Morphology**: Polarized optical microscopy for spherulite analysis

### 3.5 Mechanical Testing Protocol

- **Methods**: Tensile testing (ASTM D638) at 25°C and 60°C

- **Parameters**: Young's modulus, tensile strength, elongation at break

- **Conditioning**: 48 hr at 50% RH before testing

### 3.6 Degradation Evaluation Protocol

- **Enzymatic**: PBS buffer (pH 7.4) with 1 mg/mL lipase at 37°C

- **Hydrolytic**: PBS buffer (pH 7.4) at 60°C for accelerated testing

- **Metrics**: Mass loss (biweekly), M_n reduction (monthly), surface erosion via SEM

- **Duration**: Until >50% mass loss or 6 months

### 3.7 Biocompatibility / Biofunction Evaluation

*Provisional due to limited evidence*:

- Cytotoxicity: MTT assay with L929 fibroblasts (ISO 10993-5)

- Degradation byproduct analysis: HPLC of buffer media

### 3.8 Controls, Decision Criteria, and Statistical Comparison

- **Controls**: Neat PBS (0% isosorbide), PCL reference

- **Success Criteria**: Tg ≥ 50°C with enzymatic degradation half-life ≤ 2× PBS control

- **Statistics**: n=5 replicates; ANOVA with Tukey post-hoc (p<0.05)

## 4. Results and Evidence-Based Discussion

### 4.1 Directly Supported Expectations

- Tg elevation proportional to isosorbide content (20 mol% → ~60°C; 40 mol% → >80°C) [1, 2, 3]

- Inverse correlation between Tg and enzymatic degradation rate [4, 5]

- Crystalline domains resist degradation more than amorphous regions [6]

### 4.2 Mechanistically Inferred Expectations

- Intermediate compositions (PIS20) may balance Tg elevation (50-70°C) and degradation kinetics

- Melt-processing feasible below 240°C based on isosorbide polyesters [3]

- Hydrolytic degradation at 60°C may show autocatalytic effects in thick sections

### 4.3 Evidence Gaps and Uncertainty

- No direct data on enzymatic degradation kinetics of isosorbide-containing copolymers

- Unknown melt stability during processing at target temperatures

- Long-term (>6 month) degradation profile and byproduct accumulation uncharacterized

### 4.4 Comparison with Prior Systems

Compared to aromatic polyesters (Tg >100°C) [2, 7], isosorbide copolymers offer biodegradability but lower thermal stability. Versus PLA (Tg~55°C) [2], design targets higher service temperatures without compromising crystallinity-controlled degradation.

## 5. Risk Analysis and Optimization Path

### 5.1 Major Failure Modes

1. **Degradation suppression**: Excessive Tg elevation at >30 mol% isosorbide

2. **Processing failure**: Thermal degradation during polycondensation

3. **Crystallinity imbalance**: Slow degradation in high-crystallinity compositions

### 5.2 Optimization Pathway

- **Degradation too fast**: Increase isosorbide to 25-30 mol%; annealing to enhance crystallinity

- **Degradation too slow**: Reduce isosorbide to 10-15 mol%; incorporate low-Tg comonomers

- **Tg too low**: Increase isosorbide to 30-35 mol%; optimize catalyst for higher M_w

- **Melt instability**: Reduce polycondensation temperature; implement antioxidant additives

## 6. Conclusion

The proposed sterically hindered monomer approach shows promise for balancing heat resistance and biodegradability in polyesters. Literature confirms Tg elevation mechanisms via isosorbide incorporation but highlights degradation trade-offs requiring experimental optimization. Priority validation should focus on PIS20 compositions targeting Tg >50°C with enzymatic degradation half-life <100 days. The most critical next experiment is synthesizing the copolymer series and establishing the Tg-degradation correlation. Recommended for experimental validation with moderate risk.

## 8. References

- [1] 101002_app33935

- [2] 101016_jeurpolymj201305007

- [3] 101002_macp200900656

- [4] 101021_acsabm4c01628

- [5] 101016_jreactfunctpolym201303007

- [6] 101002_macp201000491

- [7] 101039_d5py00870k