# Design of Hydrolyzable Crosslinked Biodegradable Polyesters for Enhanced Thermal Stability with Controlled Degradation Kinetics

## Abstract

This study proposes a hydrolyzable crosslinking strategy to enhance the heat resistance of biodegradable polyesters while maintaining controlled degradation kinetics. The core hypothesis posits that strategically designed hydrolyzable crosslinks will elevate the glass transition temperature (Tg) by restricting chain mobility, while their selective hydrolysis preserves degradation rates. Literature evidence confirms crosslinking as an established method for tuning thermal properties and degradation in polyesters [1, 2, 3], though direct evidence for simultaneous Tg enhancement and degradation control remains limited. The experimental route involves synthesizing telechelic polyester oligomers with acrylate termini [2], crosslinking via Michael addition, and characterizing thermal/degradation responses. Key expected outcomes include ≥15°C Tg elevation with degradation rates within 70-130% of baseline. Major risks include excessive crosslink stability suppressing degradation and phase separation during network formation. Preliminary evidence suggests feasibility, but rigorous validation is required to balance these competing properties.

## Graphical Repeat-Unit Representation

Representative repeat-unit schematic

**Figure.** Representative polymer-structure schematic of hydrolyzable crosslinked polycaprolactone network.

Structure confidence: high. Architecture: network. Composition note: varied crosslink density. Dictionary/RDKit validation: 1/1 mapped structure(s) validated.

## 1. Introduction

### 1.1 Scientific Background

Biodegradable polyesters face inherent limitations in heat resistance (e.g., PLA's Vicat softening temperature ≈60°C [4]), restricting applications requiring thermal stability. Conventional crosslinking improves thermal properties but often severely impedes degradation kinetics [3, 5]. The design goal targets enhanced heat resistance (quantified by Tg elevation) without compromising the degradation profile essential for environmental/biomedical functionality.

### 1.2 Design Hypothesis

We hypothesize that incorporating hydrolytically labile crosslinks into polyester networks will simultaneously:

1) Increase Tg by restricting chain mobility through covalent junctions [3, 6]

2) Maintain degradation kinetics via selective cleavage of crosslinks under aqueous conditions [1, 7]

This approach is scientifically novel for its explicit decoupling of thermal enhancement from degradation suppression, potentially enabling new applications in sustainable packaging or biomedical devices.

### 1.3 Scope of This Study

Direct evidence supports crosslinking for thermal modification [3, 6] and hydrolyzable bonds for degradation control [1, 7], but no literature directly demonstrates concurrent Tg elevation and degradation maintenance using hydrolyzable crosslinks. This study will experimentally validate this balance through controlled network synthesis and characterization.

## 2. Mechanistic Rationale and Design Hypothesis

### 2.1 Mechanistic Basis

Crosslinking restricts segmental chain mobility, increasing Tg by reducing free volume [3, 6]. Hydrolyzable crosslinks (e.g., ester-containing junctions) undergo chain scission in aqueous environments, enabling network disintegration without requiring backbone degradation [1, 7]. Evidence indicates degradation rates correlate with crosslink hydrophilicity and bond lability [8, 9], while Tg elevation depends on crosslink density and junction rigidity [3, 6].

### 2.2 Evidence-Supported Design Rules

- **Thermal properties**: Crosslink density and junction rigidity primarily govern Tg elevation [3, 6]

- **Degradation**: Hydrolyzable group chemistry (e.g., ester vs. anhydride) and crosslink density control degradation rate [1, 7]

- **Mechanical properties**: Crosslinking typically increases modulus but may reduce elongation [3]

- **Biocompatibility**: Insufficient direct evidence; requires experimental validation

*Evidence gap: Quantitative relationships between crosslink hydrolysis rates and Tg elevation remain unestablished.*

### 2.3 Trade-Offs and Constraints

- **Stiffness-degradation trade-off**: Higher crosslink density increases Tg/modulus but risks slowing degradation excessively [3, 5]

- **Hydrophilicity balance**: Increased hydrophilicity accelerates degradation but may reduce thermal stability [7, 10]

- **Processing constraints**: Network formation must avoid premature hydrolysis during synthesis [7]

## 3. Materials and Methods

### 3.1 Material System and Variable Definition

- **Base oligomer**: Telechelic poly(ε-caprolactone) diol (fixed *Mₙ* = 2,000 g/mol) [2]

- **Crosslinker**: Cysteine-based trifunctional thiol (hydrolyzable ester groups) [2]

- **Variables**:

- Crosslink density (varied via [acrylate]:[thiol] stoichiometry: 1:0.8, 1:1, 1:1.2)

- Hydrolysis sensitivity (TBD: ester vs. anhydride crosslinks in Phase 2)

- **Controls**: Uncrosslinked PCL, Conventionally crosslinked polyester (non-hydrolyzable crosslinks)

### 3.2 Formulation / Sample Matrix Design

| Group | Acrylate:Thiol | Crosslink Type | Tg Target | Degradation Target |

|-------|----------------|----------------|-----------|---------------------|

| C1 | - | None (PCL control) | Baseline | Baseline |

| E1 | 1:0.8 | Hydrolyzable ester | +10°C | ≥85% of C1 |

| E2 | 1:1 | Hydrolyzable ester | +15°C | 70-130% of C1 |

| E3 | 1:1.2 | Hydrolyzable ester | +20°C | ≤115% of C1 |

| N1 | 1:1 | Non-hydrolyzable | +15°C | ≤50% of C1 |

### 3.3 Sample Preparation / Fabrication Procedure

## 1. **Oligomer synthesis**: Prepare telechelic PCL diacrylates via anionic ROP using tetrafunctional initiator and acrylate end-capping [2]

## 2. **Crosslinking**: React acrylate-terminated oligomers with trifunctional thiol crosslinker (cysteine derivative) via Michael addition:

- Solvent: Anhydrous DMF (water content <50 ppm)

- Conditions: 60°C, N₂ atmosphere, 24 hr [2]

## 3. **Film formation**: Solution-cast in PTFE molds (200 μm thickness), vacuum-dry (40°C, 48 hr)

## 4. **Critical controls**: Monitor gel fraction (>95% required); verify complete solvent removal (TGA-IR)

### 3.4 Structural and Physicochemical Characterization

- **FTIR**: Verify crosslinking (acrylate peak disappearance at 1635 cm⁻¹) [2]

- **DSC**: Measure Tg (heating rate 10°C/min, N₂ atmosphere) - *primary thermal readout* [6]

- **TGA**: Determine decomposition onset (5% weight loss) and *Tₘₐₓ* [11]

- **Swelling ratio**: Equilibrium swelling in THF (indirect crosslink density measure)

- **XRD**: Crystallinity analysis (links to degradation heterogeneity) [6]

### 3.5 Mechanical Testing Protocol

- **Tensile properties** (ASTM D638): Modulus, strength, elongation at break (dry state)

- **Prioritized comparison**: E2 vs. C1 and N1 at equivalent crosslink density

- **Wet-state testing**: After 7-day PBS immersion (37°C) to assess hydration effects

### 3.6 Degradation Evaluation Protocol

- **Accelerated hydrolysis**: PBS (pH 7.4, 37°C) + 0.1M NaOH (pH 12, 50°C for screening)

- **Monitoring**:

- Mass loss (weekly; primary degradation readout)

- GPC: Molecular weight decline

- SEM: Surface morphology changes (erosion patterns)

- pH tracking: Detect autocatalytic effects [7]

- **Termination criteria**: >50% mass loss or loss of structural integrity

### 3.7 Biocompatibility / Biofunction Evaluation

*Provisional protocol pending thermal/degradation validation:*

- Cytotoxicity: ISO 10993-5 elution test with L929 fibroblasts

- In vitro degradation: Macrophage response to degradation byproducts

### 3.8 Controls, Decision Criteria, and Statistical Comparison

- **Primary success criteria**:

- Tg(E2) ≥ Tg(C1) + 15°C

- Degradation rate(E2) = 70-130% of C1 at 28 days

- **Statistical design**: n=5 per group; ANOVA with Tukey post-hoc (α=0.05)

- **Failure thresholds**:

- Degradation rate <50% of control (excessive suppression)

- Tg increase <5°C (insufficient thermal enhancement)

## 4. Results and Evidence-Based Discussion

### 4.1 Directly Supported Expectations

- Crosslinking significantly increases Tg (literature reports +10°C to +40°C) [3, 6]

- Hydrolyzable esters degrade faster than stable crosslinks [1, 7]

- No direct evidence confirms simultaneous Tg elevation >15°C with maintained degradation kinetics

### 4.2 Mechanistically Inferred Expectations

- Moderate crosslink densities (E2 group) should balance Tg elevation and degradation maintenance

- Acidic degradation byproducts may accelerate hydrolysis autocatalytically [7]

- Phase separation likely at high crosslink densities (E3 group) [3]

### 4.3 Evidence Gaps and Uncertainty

- **Critical gaps**:

- Degradation kinetics of thiol-ester crosslinks in polyester matrices

- Long-term (>8 weeks) property retention

- Effect of crystallinity on crosslink hydrolysis [6]

- **Uncertainties**:

- pH evolution in dense crosslinked networks

- Impact of crosslink hydrolysis on mechanical integrity

### 4.4 Comparison with Prior Systems

Conventional crosslinked polyesters (N1 group) typically show degradation rates <50% of uncrosslinked controls [3, 5], while hydrolyzable crosslinks in other systems (e.g., poly(ester anhydride)s) demonstrate tunable degradation but lack Tg data [1]. PLA modifications improve heat resistance but often compromise processability and degradation [4].

## 5. Risk Analysis and Optimization Path

### 5.1 Major Failure Modes

## 1. Insufficient Tg elevation (<5°C) due to low crosslink density

## 2. Degradation oversuppression (rate <50% of control) from excessive crosslinking

## 3. Heterogeneous degradation from phase-separated networks

## 4. Premature hydrolysis during processing

### 5.2 Optimization Pathway

- **Degradation too slow**: Reduce crosslink density; switch to anhydride crosslinks

- **Degradation too fast**: Increase crosslink density; incorporate hydrophobic spacers

- **Tg too low**: Increase crosslink density; introduce rigid cyclic monomers [10]

- **Toughness too low**: Use longer oligomer chains; add plasticizers

- **Wet-state failure**: Optimize hydrophilicity via copolymerization [8, 9]

## 6. Conclusion

The hydrolyzable crosslinking strategy demonstrates strong mechanistic promise for balancing thermal enhancement and degradation control in biodegradable polyesters. While literature confirms individual mechanisms [1, 3, 7], direct evidence for simultaneous optimization remains absent. The approach is recommended for experimental validation, with moderate innovation potential (novelty in degradation-thermal decoupling). The critical next experiment involves synthesizing the E1-E3 series and quantifying Tg-degradation correlations. Success would enable thermally stable biodegradable materials for demanding applications.

## 8. References

- [1] 101002_mabi201100198

- [2] 101039_c0py00097c

- [3] 101007_s00289-017-2154-4

- [4] 101002_pat4842

- [5] 101002_macp202400067

- [6] 101007_s10965-017-1318-0

- [7] 101016_jpolymdegradstab201312031

- [8] 101002_pi6738

- [9] 101016_jeurpolymj2019109296

- [10] 101039_d3gc04489k

- [11] 101016_jpolymer201504069